# Air Quality Assessment of PurpleAir Monitoring Data Across Nebraska

Prepared for: UNMC Water, Climate, and Health Working Group
 Prepared by: Gabriel Goeken / University of Nebraska-Lincoln College of Engineering
 Study Period: February 2024 – March 2025

Introduction and Background:

One of the major environmental determinants of public health and air quality is significantly associated with high concentrations of particulate matter (PM) and volatile organic compounds (VOCs), which are risk factors for respiratory and cardiovascular diseases. The University of Nebraska Medical Center (UNMC) Water, Climate, and Health Working Group is starting a collaborative project with PurpleAir to survey ambient air quality across Nebraska through a low-cost intersectoral air quality sensor network. Given that the current air quality monitoring and interpretation is not a core competency for today's air quality team, UNMC has requested proposals for engineering assistance to analyze the PurpleAir data set and the comparison with NAAQS (National Ambient Air Quality Standards) presented by the U.S. Environmental Protection Agency (EPA). Our engineering class undertook this analysis. This study aimed to meet a number of objectives: to know where in Nebraska the average air pollutant concentrations are the highest, determine which high-polluting peak events occur, when, and at what locations. Analyze the impact of meteorological aspects, including temperature and humidity, on air quality, determine if air quality was unhealthy to vulnerable communities per the EPA air quality index (AQI) categories, and assess if the height of the sensor influences the measured air quality.

Methods:

PurpleAir sensor data was used to gather air quality data from areas throughout Nebraska. The data collection covering from February 2024 until March 2025 has time-stamped PM2.5 and PM10, volatile organic compounds, meteorological factors (temperature and relative humidity), elevation from the sensor, geo-located, etc. All data were interpreted and analyzed in Python (pandas libraries). CSV files were imported directly into a Pandas DataFrame as CSV data files. The time stamps were translated to a datetime form for temporal analyses. For the sake of data consistency, we deleted records with an invalid or missing particulate matter value as a prerequisite for analysis. All analyses were categorized according to the name of the sensor as much as possible to guarantee spatial comparability across the sites studied. Mean, median, and maximum values of VOC, PM2.5, and PM10 concentrations of each of the sensor locations. The sensors were ordered to indicate the areas with the highest average concentration of the pollutants. Maximum concentrations of the pollutants were identified in the whole dataset, and the date and sensor location of the maximum were taken. In classification. Relative humidity and temperature were classified according to client-approved values to assess the weather effect:

Humidity categories:
Low: < 50%
High: 50–80%
Very High: > 80%

Temperature categories:
Below Freezing: < 32°F
Cool: 32–50°F
Warm: 51–70°F
Hot: > 70°F

Mean PM2.5 and PM10 were calculated respectively for each section so as to find common trends. EPA AQI thresholds were subsequently used to screen for conditions considered Unhealthy for Sensitive Groups. PM2.5 concentrations of ≥ 35.5 µg/m³ and PM10 concentrations of ≥ 155 µg/m³ were flagged. Further, dates and sensor locations outside those limits were detected. Correlation analysis was done to assess the influence of sensor elevation, and summarized statistics from elevation were combined according to altitude in order to check if it affected PM concentrations. Elevation categories were also established that allowed comparison of the mean PM values from low-, medium, and high-elevation sensors.

Results and Discussion:

The detection of five sensor sites with the highest mean (mean) and median concentrations of VOC, PM2.5, and PM10. These were mostly located in urban and industrially high-traffic areas, suggesting that local emissions sources, such as vehicle traffic and close to industrial constructions, adversely affected atmospheric quality. Mean and Median Pollutant Concentration Top Sites. Peak PM, as noted, was the PM2.5 concentration recorded on 3/27/24 at the TRPHD Dawson Co. Courthouse 25, and maximum PM10 occurred on 02/18/2025 at #16 Richardson County Courthouse. VOC maxima are recorded on 06/18/2024 & 06/24/2024. High peaks in VOC numbers may reflect short-term, limited emission sources, periodicity of meteorological peaks, temperature inversions, or transportation of particles such as wildfire smoke or dust from agricultural sources. Tables 2 and 3 show that mean particulate concentrations vary by humidity and temperature groups. Higher PM2.5 and PM10 values were more commonly observed under high and very high humidity conditions, due to hygroscopic particle growth and reduced atmospheric diffusion. These results may be correlated with seasonal emission patterns and atmospheric mixing heights. Several sensors monitored PM2.5 and/or PM10 concentrations, which are Unhealthy for Sensitive Groups in the study period. These incidents took place throughout the summer. The reasons why this might have happened are due to stagnant weather, increased combustion sources, or regional pollution transport. Yes, incidents are isolated, but they probably threaten the health of such groups, such as children and the elderly, or folks with a respiratory disease. Correlational analysis of sensor elevation was in weak association with particulate concentration, as higher elevation alone indicated that elevation played an insignificant role in air quality variance for Nebraska. Grouped altitude statistics show a slightly lower average PM concentration as compared to the elevation at higher elevations, which confirms that PM is diffused in the atmosphere at a higher level and is less close to emission sources.

Conclusions:

In this report, the analysis of the PurpleAir monitoring data throughout Nebraska is presented in detail. Most locations met EPA rules for air quality, but episodic high-pollution periods happened that will need to be tracked. Respect of meteorological factors such as humidity and temperature contribute significantly to particulates’ local concentration, but elevation does a secondary role.

References (APA Format):

U.S. Environmental Protection Agency. (2024). National Ambient Air Quality Standards (NAAQS). https://www.epa.gov/criteria-air-pollutants/naaqs-table

U.S. Environmental Protection Agency. (2024). National Ambient Air Quality Standards (NAAQS). https://www.epa.gov/criteria-air-pollutants/naaqs-table

U.S. Environmental Protection Agency. (2024). Air Quality Index (AQI) basics. https://www.airnow.gov/aqi/aqi-basics/

PurpleAir. (2024). PurpleAir community and data resources. https://community.purpleair.com

UNMC Water, Climate and Health Working Group. (2024). Project background materials for PurpleAir air quality analysis. https://www.unmc.edu

UNMC Water, Climate and Health Working Group. (2024). Project background materials.